Intelligent Data Center Resource and

Storage Optimization System

Operating Systems — Course Assignment Report

Language Used: C  |  Deliverable: Pseudocode, Implementation, Sample I/O, Analysis

# 1. Problem Analysis and Assumptions

## 1.1 Problem Analysis

A large cloud data center runs applications from banking, e-commerce, healthcare and analytics domains on shared infrastructure. Performance degrades when CPU scheduling, memory allocation, file storage and disk I/O are not matched to workload characteristics. This project builds a single analytical system that takes one common workload as input, evaluates it against multiple classical operating-system algorithms in each subsystem (process scheduling, deadlock safety, memory management, file allocation, disk scheduling), and recommends the best-performing combination using measurable criteria.

## 1.2 Assumptions

- All burst times, arrival times and resource requests are known in advance (as typically assumed for offline/simulation-based scheduling analysis).

- A single CPU core is modelled; context-switch overhead is assumed negligible (0 ms) for clarity of comparison.

- Priority values: lower numeric value = higher priority.

- Round Robin time quantum = 3 ms (chosen empirically for the given burst-time range).

- Banker's Algorithm is used for both deadlock avoidance (pre-request check) and safety-state detection using the classical Allocation / Max / Available matrices.

- Memory sizes are expressed in KB; page size = 1024 KB; disk block size is treated as one abstract unit for file allocation.

- For file allocation, a disk of 40 blocks is simulated with 10 blocks pre-occupied in a scattered pattern to realistically demonstrate external fragmentation under contiguous allocation.

- Disk cylinders range from 0–99 (100 cylinders), satisfying the 50–100 cylinder requirement; the initial head position is 50.

- The same input dataset (processes, resource matrices, memory sizes, files, disk requests) is reused across every technique in a component so that comparisons are fair.

## 1.3 System Modules

| Component | Techniques Implemented | Key Metrics |
| --- | --- | --- |
| Process Management | FCFS, SJF (NP), Priority (NP), Round Robin | Waiting Time, Turnaround Time, Response Time, CPU Utilization |
| Deadlock Analysis | Banker's Algorithm | Safe / Unsafe State, Safe Sequence |
| Memory Management | Paging, Segmentation | Internal / External Fragmentation, Memory Utilization |
| File Allocation | Contiguous, Linked, Indexed | Space Utilization, Access Cost, Fragmentation |
| Disk Scheduling | FCFS, SSTF, SCAN, C-SCAN, LOOK, C-LOOK | Total Head Movement, Average Seek Time |

# 2. Algorithm / Pseudocode

## 2.1 CPU Scheduling

### FCFS

Sort processes by Arrival Time

time = 0

for each process p in sorted order:

if time < p.AT: time = p.AT

p.ResponseTime = time - p.AT

time = time + p.BT

p.CompletionTime = time

p.TurnaroundTime = p.CT - p.AT

p.WaitingTime = p.TAT - p.BT

### SJF (Non-Preemptive)

while processes remain:

among processes with AT <= current_time and not done,

pick the one with SMALLEST Burst Time

if none available: time++ ; continue

Response = time - AT ; time += BT

Completion = time ; Turnaround = CT - AT ; Waiting = TAT - BT

mark done

### Priority Scheduling (Non-Preemptive)

while processes remain:

among processes with AT <= current_time and not done,

pick the one with SMALLEST priority number (highest priority)

if none available: time++ ; continue

Response = time - AT ; time += BT

Completion = time ; Turnaround = CT - AT ; Waiting = TAT - BT

mark done

### Round Robin (Quantum = q)

ready_queue = empty

time = first arrival time ; enqueue all processes that have arrived

while processes remain:

if ready_queue empty: jump time to next arrival ; enqueue it

p = dequeue()

if p not started: p.Response = time - p.AT

run = min(p.remaining, q)

time += run ; p.remaining -= run

enqueue any new arrivals that occurred during 'run'

if p.remaining > 0: enqueue(p)

else: p.Completion = time ; compute TAT, WT

## 2.2 Deadlock Detection / Avoidance — Banker's Algorithm

Need[i][j] = Max[i][j] - Allocation[i][j]

Work = Available ; Finish[i] = false for all i

repeat:

progress = false

for each process i with Finish[i] == false:

if Need[i] <= Work (element-wise):

Work = Work + Allocation[i]

Finish[i] = true

add i to Safe_Sequence

progress = true

until progress == false

if all Finish[i] == true: SYSTEM IS SAFE, print Safe_Sequence

else: SYSTEM IS UNSAFE (deadlock risk) for remaining processes

## 2.3 Memory Management

### Paging

for each process p:

pages_needed = ceil(p.size / page_size)

frame_alloc  = pages_needed * page_size

internal_fragmentation = frame_alloc - p.size

Total_frames_needed = sum(pages_needed)

if Total_frames_needed > available_frames: flag insufficient frames

Memory_Utilization = (sum(p.size) / (page_size * available_frames)) * 100

### Segmentation (First-Fit on Variable Partitions)

for each segment s (in given order):

for each free partition part (in given order):

if part.size >= s.size:

allocate s to part

external_fragmentation += (part.size - s.size)

mark part as used ; break

if not allocated: report "segment cannot be placed"

Memory_Utilization = (sum(allocated segment sizes) / sum(partition sizes)) * 100

## 2.4 File Allocation Strategies

CONTIGUOUS:

for each file f: find first run of free blocks >= f.size on the disk bitmap

if found: allocate contiguous run

else: allocation FAILS (external fragmentation)

LINKED:

for each file f: allocate f.size blocks from ANYWHERE on disk (order irrelevant)

each block stores a pointer to the next block (no separate index block)

Average random-access cost = (f.size + 1) / 2 blocks (must traverse the chain)

INDEXED:

for each file f: allocate 1 index block + f.size data blocks (blocks anywhere)

index block holds pointers to every data block -> direct access

Overhead = 1 extra block per file

## 2.5 Disk Scheduling

FCFS:    Service requests strictly in arrival order.

SSTF:    Repeatedly service the request with the SHORTEST seek distance

from the current head position.

SCAN:    Move head in one direction servicing all requests until the

end of the disk, then reverse direction and service the rest.

C-SCAN:  Move head in one direction servicing requests until the end,

then jump immediately to the opposite end and continue the

SAME direction (treats disk as circular).

LOOK:    Like SCAN, but reverses at the LAST request in that direction

instead of travelling to the physical end of the disk.

C-LOOK:  Like C-SCAN, but jumps to the SMALLEST/LARGEST pending request

instead of the physical end of the disk.

For each algorithm: Total_Head_Movement = sum of |cylinder(i) - cylinder(i-1)|

Average_Seek_Time   = Total_Head_Movement / number_of_requests

# 3. Implementation (C Language Source Code)

The complete C implementation is provided below (also included as datacenter.c for GitHub upload). It compiles cleanly with gcc and produces the sample output shown in Section 4 for a single embedded workload, so results are directly comparable across techniques within each module.

/* =====================================================================

INTELLIGENT DATA CENTER RESOURCE AND STORAGE OPTIMIZATION SYSTEM

-----------------------------------------------------------------

Modules:

1. CPU Scheduling      : FCFS, SJF (NP), Priority (NP), Round Robin

2. Deadlock Analysis    : Banker's Algorithm (Safety Check)

3. Memory Management    : Paging & Segmentation

4. File Allocation      : Contiguous, Linked, Indexed

5. Disk Scheduling      : FCFS, SSTF, SCAN, C-SCAN, LOOK, C-LOOK

All modules run on ONE common workload so results are comparable.

===================================================================== */

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <math.h>

#define MAXP 20

#define MAXR 5

#define MAXF 10

#define MAXD 20

#define MAXCYL 200

/* ---------------------------------------------------------------------

================  1. CPU SCHEDULING MODULE  =========================

--------------------------------------------------------------------- */

typedef struct {

int pid, at, bt, pr;      /* pid, arrival, burst, priority(1=highest) */

int ct, wt, tat, rt;      /* completion, waiting, turnaround, response */

int remaining;

int started;

} Process;

Process orig[MAXP];

int nproc;

void loadProcesses(Process p[]) {

memcpy(p, orig, sizeof(Process) * nproc);

for (int i = 0; i < nproc; i++) {

p[i].remaining = p[i].bt;

p[i].started = 0;

p[i].ct = p[i].wt = p[i].tat = p[i].rt = 0;

}

}

void printGantt(int order[], int n) {

printf("Execution order (PID sequence): ");

for (int i = 0; i < n; i++) printf("P%d ", order[i]);

printf("\n");

}

void printSummary(const char *name, Process p[], int n, int totalTime) {

double avgWT = 0, avgTAT = 0, avgRT = 0;

printf("\n----- %s : Result Table -----\n", name);

printf("PID\tAT\tBT\tCT\tWT\tTAT\tRT\n");

for (int i = 0; i < n; i++) {

printf("P%d\t%d\t%d\t%d\t%d\t%d\t%d\n",

p[i].pid, p[i].at, p[i].bt, p[i].ct, p[i].wt, p[i].tat, p[i].rt);

avgWT += p[i].wt; avgTAT += p[i].tat; avgRT += p[i].rt;

}

avgWT /= n; avgTAT /= n; avgRT /= n;

int busy = 0;

for (int i = 0; i < n; i++) busy += p[i].bt;

double cpuUtil = (100.0 * busy) / totalTime;

printf("Average Waiting Time    : %.2f\n", avgWT);

printf("Average Turnaround Time : %.2f\n", avgTAT);

printf("Average Response Time   : %.2f\n", avgRT);

printf("CPU Utilization         : %.2f %%\n", cpuUtil);

}

/* ---- FCFS ---- */

void fcfs() {

Process p[MAXP];

loadProcesses(p);

/* sort by arrival time (stable) */

for (int i = 0; i < nproc - 1; i++)

for (int j = 0; j < nproc - 1 - i; j++)

if (p[j].at > p[j + 1].at) { Process t = p[j]; p[j] = p[j + 1]; p[j + 1] = t; }

int time = 0, order[MAXP];

for (int i = 0; i < nproc; i++) {

if (time < p[i].at) time = p[i].at;

p[i].rt = time - p[i].at;

time += p[i].bt;

p[i].ct = time;

p[i].tat = p[i].ct - p[i].at;

p[i].wt = p[i].tat - p[i].bt;

order[i] = p[i].pid;

}

printGantt(order, nproc);

printSummary("FCFS", p, nproc, time);

}

/* ---- SJF Non-Preemptive ---- */

void sjf() {

Process p[MAXP];

loadProcesses(p);

int done[MAXP] = {0}, completed = 0, time = 0, order[MAXP], oi = 0;

while (completed < nproc) {

int idx = -1, minBT = 1e9;

for (int i = 0; i < nproc; i++)

if (!done[i] && p[i].at <= time && p[i].bt < minBT) { minBT = p[i].bt; idx = i; }

if (idx == -1) { time++; continue; }

p[idx].rt = time - p[idx].at;

time += p[idx].bt;

p[idx].ct = time;

p[idx].tat = p[idx].ct - p[idx].at;

p[idx].wt = p[idx].tat - p[idx].bt;

done[idx] = 1; completed++;

order[oi++] = p[idx].pid;

}

printGantt(order, oi);

printSummary("SJF (Non-Preemptive)", p, nproc, time);

}

/* ---- Priority Non-Preemptive (lower number = higher priority) ---- */

void priorityScheduling() {

Process p[MAXP];

loadProcesses(p);

int done[MAXP] = {0}, completed = 0, time = 0, order[MAXP], oi = 0;

while (completed < nproc) {

int idx = -1, best = 1e9;

for (int i = 0; i < nproc; i++)

if (!done[i] && p[i].at <= time && p[i].pr < best) { best = p[i].pr; idx = i; }

if (idx == -1) { time++; continue; }

p[idx].rt = time - p[idx].at;

time += p[idx].bt;

p[idx].ct = time;

p[idx].tat = p[idx].ct - p[idx].at;

p[idx].wt = p[idx].tat - p[idx].bt;

done[idx] = 1; completed++;

order[oi++] = p[idx].pid;

}

printGantt(order, oi);

printSummary("Priority Scheduling (Non-Preemptive)", p, nproc, time);

}

/* ---- Round Robin ---- */

void roundRobin(int quantum) {

Process p[MAXP];

loadProcesses(p);

int queue[500], front = 0, rear = 0, inQueue[MAXP] = {0};

int time = 0, completed = 0, order[500], oi = 0;

/* sort by arrival for insertion handling */

int arr_order[MAXP];

for (int i = 0; i < nproc; i++) arr_order[i] = i;

for (int i = 0; i < nproc - 1; i++)

for (int j = 0; j < nproc - 1 - i; j++)

if (p[arr_order[j]].at > p[arr_order[j + 1]].at) {

int t = arr_order[j]; arr_order[j] = arr_order[j + 1]; arr_order[j + 1] = t;

}

int ai = 0;

time = p[arr_order[0]].at;

while (ai < nproc && p[arr_order[ai]].at <= time) { queue[rear++] = arr_order[ai]; inQueue[arr_order[ai]] = 1; ai++; }

while (completed < nproc) {

if (front == rear) { /* queue empty, jump to next arrival */

if (ai < nproc) { time = p[arr_order[ai]].at; queue[rear++] = arr_order[ai]; inQueue[arr_order[ai]] = 1; ai++; }

else break;

continue;

}

int idx = queue[front++];

if (!p[idx].started) { p[idx].rt = time - p[idx].at; p[idx].started = 1; }

int run = (p[idx].remaining < quantum) ? p[idx].remaining : quantum;

time += run;

p[idx].remaining -= run;

order[oi++] = p[idx].pid;

/* enqueue any new arrivals during this run */

while (ai < nproc && p[arr_order[ai]].at <= time) { queue[rear++] = arr_order[ai]; inQueue[arr_order[ai]] = 1; ai++; }

if (p[idx].remaining > 0) queue[rear++] = idx;

else { p[idx].ct = time; p[idx].tat = p[idx].ct - p[idx].at; p[idx].wt = p[idx].tat - p[idx].bt; completed++; }

}

printGantt(order, oi);

printSummary("Round Robin (Quantum = 3)", p, nproc, time);

}

/* ---------------------------------------------------------------------

================  2. DEADLOCK ANALYSIS (BANKER'S) ===================

--------------------------------------------------------------------- */

int bnk_n, bnk_m;

int alloc_[MAXP][MAXR], maxm[MAXP][MAXR], need[MAXP][MAXR], avail[MAXR];

void bankersAlgorithm() {

printf("\n----- Banker's Algorithm : Deadlock Avoidance / Safety Check -----\n");

for (int i = 0; i < bnk_n; i++)

for (int j = 0; j < bnk_m; j++)

need[i][j] = maxm[i][j] - alloc_[i][j];

printf("Process\tAllocation\tMax\t\tNeed\n");

for (int i = 0; i < bnk_n; i++) {

printf("P%d\t", i);

for (int j = 0; j < bnk_m; j++) printf("%d ", alloc_[i][j]);

printf("\t");

for (int j = 0; j < bnk_m; j++) printf("%d ", maxm[i][j]);

printf("\t");

for (int j = 0; j < bnk_m; j++) printf("%d ", need[i][j]);

printf("\n");

}

printf("Available: ");

for (int j = 0; j < bnk_m; j++) printf("%d ", avail[j]);

printf("\n");

int work[MAXR], finish[MAXP] = {0}, safeSeq[MAXP], si = 0;

for (int j = 0; j < bnk_m; j++) work[j] = avail[j];

int progress = 1;

while (progress) {

progress = 0;

for (int i = 0; i < bnk_n; i++) {

if (finish[i]) continue;

int ok = 1;

for (int j = 0; j < bnk_m; j++) if (need[i][j] > work[j]) { ok = 0; break; }

if (ok) {

for (int j = 0; j < bnk_m; j++) work[j] += alloc_[i][j];

finish[i] = 1;

safeSeq[si++] = i;

progress = 1;

}

}

}

int safe = 1;

for (int i = 0; i < bnk_n; i++) if (!finish[i]) safe = 0;

if (safe) {

printf("Result: SYSTEM IS IN A SAFE STATE\n");

printf("Safe Sequence: < ");

for (int i = 0; i < si; i++) printf("P%d ", safeSeq[i]);

printf(">\n");

} else {

printf("Result: SYSTEM IS IN AN UNSAFE STATE (Deadlock possible)\n");

printf("Processes that CANNOT complete: ");

for (int i = 0; i < bnk_n; i++) if (!finish[i]) printf("P%d ", i);

printf("\n");

}

}

/* ---------------------------------------------------------------------

================  3. MEMORY MANAGEMENT  ==============================

--------------------------------------------------------------------- */

void pagingAnalysis(int procSize[], int n, int pageSize, int totalFrames) {

printf("\n----- Paging Analysis -----\n");

printf("Page Size = %d KB | Total Frames Available = %d (%d KB)\n",

pageSize, totalFrames, pageSize * totalFrames);

printf("Proc\tSize(KB)\tPagesNeeded\tFrameAlloc(KB)\tInternalFrag(KB)\n");

int totalPages = 0, totalIntFrag = 0, totalUsed = 0;

for (int i = 0; i < n; i++) {

int pages = (int)ceil((double)procSize[i] / pageSize);

int alloc = pages * pageSize;

int frag = alloc - procSize[i];

printf("P%d\t%d\t\t%d\t\t%d\t\t%d\n", i + 1, procSize[i], pages, alloc, frag);

totalPages += pages; totalIntFrag += frag; totalUsed += procSize[i];

}

double memUtil = 0;

int totalMem = pageSize * totalFrames;

if (totalPages <= totalFrames)

memUtil = (100.0 * totalUsed) / totalMem;

else

memUtil = (100.0 * totalUsed) / totalMem; /* still shows demand vs supply */

printf("Total Frames Needed     : %d %s\n", totalPages,

(totalPages > totalFrames) ? "(EXCEEDS available frames -> page-in/out & possible page faults)" : "(fits in available frames)");

printf("Total Internal Fragmentation : %d KB\n", totalIntFrag);

printf("Memory Utilization           : %.2f %%\n", memUtil);

}

void segmentationAnalysis(int segSize[], int n, int partition[], int npart) {

printf("\n----- Segmentation Analysis (First-Fit on Variable Partitions) -----\n");

int used[MAXP] = {0};

int allocatedPart[MAXP]; for (int i = 0; i < n; i++) allocatedPart[i] = -1;

int partUsed[MAXP] = {0};

int totalExtFrag = 0, totalAllocated = 0, unallocated = 0;

for (int i = 0; i < n; i++) {

for (int j = 0; j < npart; j++) {

if (!partUsed[j] && partition[j] >= segSize[i]) {

allocatedPart[i] = j;

partUsed[j] = 1;

totalExtFrag += (partition[j] - segSize[i]);

totalAllocated += segSize[i];

break;

}

}

if (allocatedPart[i] == -1) unallocated++;

}

printf("Segment\tSize(KB)\tPartitionAlloc\tPartSize(KB)\tExtFrag(KB)\n");

for (int i = 0; i < n; i++) {

if (allocatedPart[i] != -1)

printf("S%d\t%d\t\tPart-%d\t\t%d\t\t%d\n", i + 1, segSize[i], allocatedPart[i] + 1,

partition[allocatedPart[i]], partition[allocatedPart[i]] - segSize[i]);

else

printf("S%d\t%d\t\tNOT ALLOCATED\t-\t\t-\n", i + 1, segSize[i]);

}

int totalPartMem = 0;

for (int i = 0; i < npart; i++) totalPartMem += partition[i];

printf("Total External Fragmentation : %d KB\n", totalExtFrag);

printf("Unallocated Segments          : %d\n", unallocated);

printf("Memory Utilization            : %.2f %%\n", (100.0 * totalAllocated) / totalPartMem);

}

/* ---------------------------------------------------------------------

================  4. FILE ALLOCATION STRATEGIES  =====================

--------------------------------------------------------------------- */

void fileAllocationAnalysis(int fileBlocks[], int nf, int totalDiskBlocks, int preUsed) {

printf("\n----- File Allocation Strategy Comparison -----\n");

printf("Total Disk Blocks = %d | Pre-occupied (fragmented) blocks = %d\n", totalDiskBlocks, preUsed);

/* Simulate a disk with 'preUsed' blocks scattered (every alternate block busy)

to create realistic external fragmentation for contiguous allocation. */

int disk[MAXCYL] = {0};

for (int i = 0; i < preUsed && i * 2 < totalDiskBlocks; i++) disk[i * 2] = 1; /* scattered holes */

int freeBlocksTotal = 0;

for (int i = 0; i < totalDiskBlocks; i++) if (!disk[i]) freeBlocksTotal++;

/* ---- Contiguous Allocation ---- */

int diskC[MAXCYL]; memcpy(diskC, disk, sizeof(disk));

int contigSuccess = 0, contigFail = 0, contigBlocksUsed = 0;

for (int f = 0; f < nf; f++) {

int need = fileBlocks[f], run = 0, start = -1;

for (int i = 0; i < totalDiskBlocks; i++) {

if (!diskC[i]) { if (run == 0) start = i; run++; if (run == need) break; }

else run = 0;

}

if (run == need) {

for (int i = start; i < start + need; i++) diskC[i] = 1;

contigSuccess++; contigBlocksUsed += need;

} else contigFail++;

}

/* ---- Linked Allocation (any free block works, +0 extra block overhead,

pointer stored inside block so no separate block, but access is sequential) ---- */

int totalFileBlocks = 0;

for (int f = 0; f < nf; f++) totalFileBlocks += fileBlocks[f];

int linkedSuccess = (totalFileBlocks <= freeBlocksTotal) ? nf : 0;

int linkedBlocksUsed = totalFileBlocks; /* no contiguity required */

/* ---- Indexed Allocation (needs +1 index block per file) ---- */

int indexedBlocksUsed = totalFileBlocks + nf;

int indexedSuccess = (indexedBlocksUsed <= freeBlocksTotal) ? nf : 0;

printf("\nMethod\t\tFilesAllocated\tBlocksUsed\tOverheadBlocks\tAvgAccessCost(blocks)\n");

printf("Contiguous\t%d/%d\t\t%d\t\t0\t\t1 (direct, seq. fast)\n", contigSuccess, nf, contigBlocksUsed);

double avgLinked = 0; for (int f = 0; f < nf; f++) avgLinked += (fileBlocks[f] + 1) / 2.0;

avgLinked /= nf;

printf("Linked\t\t%d/%d\t\t%d\t\t0\t\t%.2f (must traverse chain)\n", linkedSuccess, nf, linkedBlocksUsed, avgLinked);

printf("Indexed\t\t%d/%d\t\t%d\t\t%d\t\t2 (index + 1 data block)\n", indexedSuccess, nf, indexedBlocksUsed, nf);

printf("\nContiguous: %s due to external fragmentation (%d holes pre-occupied on disk).\n",

(contigFail > 0) ? "SOME FILES FAILED to allocate" : "all files allocated successfully",

preUsed);

printf("Linked   : No external fragmentation; scattered blocks are fine; poor random access.\n");

printf("Indexed  : No external fragmentation; %d extra index blocks used; good random access.\n", nf);

}

/* ---------------------------------------------------------------------

================  5. DISK SCHEDULING  ================================

--------------------------------------------------------------------- */

int cmpInt(const void *a, const void *b) { return (*(int *)a - *(int *)b); }

void diskFCFS(int req[], int n, int head) {

int total = 0, cur = head;

printf("FCFS  Order: %d", head);

for (int i = 0; i < n; i++) { total += abs(req[i] - cur); cur = req[i]; printf(" -> %d", cur); }

printf("\nFCFS  Total Head Movement = %d | Avg Seek = %.2f\n", total, (double)total / n);

}

void diskSSTF(int req[], int n, int head) {

int visited[MAXD] = {0}, total = 0, cur = head;

printf("SSTF  Order: %d", head);

for (int c = 0; c < n; c++) {

int best = -1, bestDist = 1e9;

for (int i = 0; i < n; i++)

if (!visited[i] && abs(req[i] - cur) < bestDist) { bestDist = abs(req[i] - cur); best = i; }

visited[best] = 1; total += bestDist; cur = req[best]; printf(" -> %d", cur);

}

printf("\nSSTF  Total Head Movement = %d | Avg Seek = %.2f\n", total, (double)total / n);

}

void diskSCAN(int req[], int n, int head, int maxCyl, int dirUp) {

int r[MAXD]; memcpy(r, req, sizeof(int) * n);

qsort(r, n, sizeof(int), cmpInt);

int total = 0, cur = head;

printf("SCAN  Order: %d", head);

if (dirUp) {

for (int i = 0; i < n; i++) if (r[i] >= head) { total += abs(r[i] - cur); cur = r[i]; printf(" -> %d", cur); }

total += abs((maxCyl - 1) - cur); cur = maxCyl - 1; printf(" -> %d(end)", cur);

for (int i = n - 1; i >= 0; i--) if (r[i] < head) { total += abs(r[i] - cur); cur = r[i]; printf(" -> %d", cur); }

} else {

for (int i = n - 1; i >= 0; i--) if (r[i] <= head) { total += abs(r[i] - cur); cur = r[i]; printf(" -> %d", cur); }

total += abs(0 - cur); cur = 0; printf(" -> 0(end)");

for (int i = 0; i < n; i++) if (r[i] > head) { total += abs(r[i] - cur); cur = r[i]; printf(" -> %d", cur); }

}

printf("\nSCAN  Total Head Movement = %d | Avg Seek = %.2f\n", total, (double)total / n);

}

void diskCSCAN(int req[], int n, int head, int maxCyl) {

int r[MAXD]; memcpy(r, req, sizeof(int) * n);

qsort(r, n, sizeof(int), cmpInt);

int total = 0, cur = head;

printf("C-SCAN Order: %d", head);

for (int i = 0; i < n; i++) if (r[i] >= head) { total += abs(r[i] - cur); cur = r[i]; printf(" -> %d", cur); }

total += abs((maxCyl - 1) - cur); cur = maxCyl - 1; printf(" -> %d(end)", cur);

total += (maxCyl - 1); cur = 0; printf(" -> 0(jump)");

for (int i = 0; i < n; i++) if (r[i] < head) { total += abs(r[i] - cur); cur = r[i]; printf(" -> %d", cur); }

printf("\nC-SCAN Total Head Movement = %d | Avg Seek = %.2f\n", total, (double)total / n);

}

void diskLOOK(int req[], int n, int head, int dirUp) {

int r[MAXD]; memcpy(r, req, sizeof(int) * n);

qsort(r, n, sizeof(int), cmpInt);

int total = 0, cur = head;

printf("LOOK  Order: %d", head);

if (dirUp) {

for (int i = 0; i < n; i++) if (r[i] >= head) { total += abs(r[i] - cur); cur = r[i]; printf(" -> %d", cur); }

for (int i = n - 1; i >= 0; i--) if (r[i] < head) { total += abs(r[i] - cur); cur = r[i]; printf(" -> %d", cur); }

} else {

for (int i = n - 1; i >= 0; i--) if (r[i] <= head) { total += abs(r[i] - cur); cur = r[i]; printf(" -> %d", cur); }

for (int i = 0; i < n; i++) if (r[i] > head) { total += abs(r[i] - cur); cur = r[i]; printf(" -> %d", cur); }

}

printf("\nLOOK  Total Head Movement = %d | Avg Seek = %.2f\n", total, (double)total / n);

}

void diskCLOOK(int req[], int n, int head) {

int r[MAXD]; memcpy(r, req, sizeof(int) * n);

qsort(r, n, sizeof(int), cmpInt);

int total = 0, cur = head;

printf("C-LOOK Order: %d", head);

int firstAbove = -1;

for (int i = 0; i < n; i++) if (r[i] >= head) { total += abs(r[i] - cur); cur = r[i]; printf(" -> %d", cur); }

/* jump to smallest request (not to cylinder 0) */

int smallest = r[0];

if (cur != smallest && smallest < head) {

total += abs(cur - smallest);

cur = smallest;

}

for (int i = 0; i < n; i++) if (r[i] < head) { if(r[i]!=cur){total += abs(r[i]-cur); cur=r[i];} printf(" -> %d", cur); }

printf("\nC-LOOK Total Head Movement = %d | Avg Seek = %.2f\n", total, (double)total / n);

}

/* =====================================================================

MAIN

===================================================================== */

int main() {

printf("=====================================================================\n");

printf(" INTELLIGENT DATA CENTER RESOURCE AND STORAGE OPTIMIZATION SYSTEM\n");

printf("=====================================================================\n");

/* ---------------- SAMPLE WORKLOAD (single dataset used by ALL modules) --------------- */

/* ---- 1. Process set: 10 processes ---- */

nproc = 10;

int PID[10]  = {1,2,3,4,5,6,7,8,9,10};

int AT[10]   = {0,1,2,3,4,5,6,7,8,9};

int BT[10]   = {5,3,8,6,4,2,7,5,3,6};

int PR[10]   = {2,1,3,2,1,4,3,2,1,4};

for (int i = 0; i < nproc; i++) {

orig[i].pid = PID[i]; orig[i].at = AT[i]; orig[i].bt = BT[i]; orig[i].pr = PR[i];

}

printf("\n============== MODULE 1: CPU SCHEDULING ==============\n");

printf("Input Processes (PID, ArrivalTime, BurstTime, Priority):\n");

for (int i = 0; i < nproc; i++)

printf("P%d : AT=%d BT=%d Priority=%d\n", orig[i].pid, orig[i].at, orig[i].bt, orig[i].pr);

fcfs();

sjf();

priorityScheduling();

roundRobin(3);

/* ---- 2. Deadlock: Banker's Algorithm dataset (classic 5-process/3-resource) ---- */

printf("\n============== MODULE 2: DEADLOCK DETECTION / AVOIDANCE ==============\n");

bnk_n = 5; bnk_m = 3;

int A[5][3] = {{0,1,0},{2,0,0},{3,0,2},{2,1,1},{0,0,2}};

int M[5][3] = {{7,5,3},{3,2,2},{9,0,2},{2,2,2},{4,3,3}};

int AV[3] = {3,3,2};

for (int i = 0; i < bnk_n; i++)

for (int j = 0; j < bnk_m; j++) { alloc_[i][j] = A[i][j]; maxm[i][j] = M[i][j]; }

for (int j = 0; j < bnk_m; j++) avail[j] = AV[j];

bankersAlgorithm();

/* ---- 3. Memory Management dataset ---- */

printf("\n============== MODULE 3: MEMORY MANAGEMENT ==============\n");

int procSize[5] = {2200, 3600, 1200, 4800, 1500}; /* KB, reuse for paging & segmentation */

pagingAnalysis(procSize, 5, 1024, 12);            /* page size 1024KB, 12 frames = 12288KB */

int partitions[5] = {2500, 4000, 1300, 5000, 1600}; /* variable partitions, KB */

segmentationAnalysis(procSize, 5, partitions, 5);

/* ---- 4. File Allocation dataset ---- */

printf("\n============== MODULE 4: FILE ALLOCATION STRATEGIES ==============\n");

int fileBlocks[6] = {3, 5, 2, 8, 4, 6}; /* 6 files, sizes in blocks */

fileAllocationAnalysis(fileBlocks, 6, 40, 10); /* 40 disk blocks, 10 pre-occupied (fragmented) */

/* ---- 5. Disk Scheduling dataset ---- */

printf("\n============== MODULE 5: DISK SCHEDULING ==============\n");

int requests[12] = {82, 95, 30, 63, 12, 70, 25, 48, 90, 5, 77, 34};

int headStart = 50, maxCyl = 100;

printf("Requests (12): "); for (int i = 0; i < 12; i++) printf("%d ", requests[i]); printf("\n");

printf("Initial Head Position = %d | Cylinders = 0-%d\n\n", headStart, maxCyl - 1);

diskFCFS(requests, 12, headStart);

printf("\n");

diskSSTF(requests, 12, headStart);

printf("\n");

diskSCAN(requests, 12, headStart, maxCyl, 1);

printf("\n");

diskCSCAN(requests, 12, headStart, maxCyl);

printf("\n");

diskLOOK(requests, 12, headStart, 1);

printf("\n");

diskCLOOK(requests, 12, headStart);

printf("\n=====================================================================\n");

printf(" END OF ANALYSIS\n");

printf("=====================================================================\n");

return 0;

}

# 4. Sample Input and Output

## 4.1 Sample Input Dataset

Process Set (10 processes):

| PID | Arrival Time | Burst Time | Priority |
| --- | --- | --- | --- |
| P1 | 0 | 5 | 2 |
| P2 | 1 | 3 | 1 |
| P3 | 2 | 8 | 3 |
| P4 | 3 | 6 | 2 |
| P5 | 4 | 4 | 1 |
| P6 | 5 | 2 | 4 |
| P7 | 6 | 7 | 3 |
| P8 | 7 | 5 | 2 |
| P9 | 8 | 3 | 1 |
| P10 | 9 | 6 | 4 |

Banker's Algorithm data (5 processes, 3 resource types A, B, C — Available = 3 3 2):

| Process | Allocation (A B C) | Max (A B C) |
| --- | --- | --- |
| P0 | 0 1 0 | 7 5 3 |
| P1 | 2 0 0 | 3 2 2 |
| P2 | 3 0 2 | 9 0 2 |
| P3 | 2 1 1 | 2 2 2 |
| P4 | 0 0 2 | 4 3 3 |

Memory (5 processes/segments, KB): 2200, 3600, 1200, 4800, 1500  |  Page size = 1024 KB, 12 frames  |  Partitions: 2500, 4000, 1300, 5000, 1600

Files (6 files, in blocks): 3, 5, 2, 8, 4, 6  |  Disk = 40 blocks, 10 pre-occupied (fragmented)

Disk requests (12, initial head = 50, cylinders 0–99): 82, 95, 30, 63, 12, 70, 25, 48, 90, 5, 77, 34

## 4.2 Program Output (actual run)

=====================================================================

INTELLIGENT DATA CENTER RESOURCE AND STORAGE OPTIMIZATION SYSTEM

=====================================================================

============== MODULE 1: CPU SCHEDULING ==============

Input Processes (PID, ArrivalTime, BurstTime, Priority):

P1 : AT=0 BT=5 Priority=2

P2 : AT=1 BT=3 Priority=1

P3 : AT=2 BT=8 Priority=3

P4 : AT=3 BT=6 Priority=2

P5 : AT=4 BT=4 Priority=1

P6 : AT=5 BT=2 Priority=4

P7 : AT=6 BT=7 Priority=3

P8 : AT=7 BT=5 Priority=2

P9 : AT=8 BT=3 Priority=1

P10 : AT=9 BT=6 Priority=4

Execution order (PID sequence): P1 P2 P3 P4 P5 P6 P7 P8 P9 P10

----- FCFS : Result Table -----

PID	AT	BT	CT	WT	TAT	RT

P1	0	5	5	0	5	0

P2	1	3	8	4	7	4

P3	2	8	16	6	14	6

P4	3	6	22	13	19	13

P5	4	4	26	18	22	18

P6	5	2	28	21	23	21

P7	6	7	35	22	29	22

P8	7	5	40	28	33	28

P9	8	3	43	32	35	32

P10	9	6	49	34	40	34

Average Waiting Time    : 17.80

Average Turnaround Time : 22.70

Average Response Time   : 17.80

CPU Utilization         : 100.00 %

Execution order (PID sequence): P1 P6 P2 P9 P5 P8 P4 P10 P7 P3

----- SJF (Non-Preemptive) : Result Table -----

PID	AT	BT	CT	WT	TAT	RT

P1	0	5	5	0	5	0

P2	1	3	10	6	9	6

P3	2	8	49	39	47	39

P4	3	6	28	19	25	19

P5	4	4	17	9	13	9

P6	5	2	7	0	2	0

P7	6	7	41	28	35	28

P8	7	5	22	10	15	10

P9	8	3	13	2	5	2

P10	9	6	34	19	25	19

Average Waiting Time    : 13.20

Average Turnaround Time : 18.10

Average Response Time   : 13.20

CPU Utilization         : 100.00 %

Execution order (PID sequence): P1 P2 P5 P9 P4 P8 P3 P7 P6 P10

----- Priority Scheduling (Non-Preemptive) : Result Table -----

PID	AT	BT	CT	WT	TAT	RT

P1	0	5	5	0	5	0

P2	1	3	8	4	7	4

P3	2	8	34	24	32	24

P4	3	6	21	12	18	12

P5	4	4	12	4	8	4

P6	5	2	43	36	38	36

P7	6	7	41	28	35	28

P8	7	5	26	14	19	14

P9	8	3	15	4	7	4

P10	9	6	49	34	40	34

Average Waiting Time    : 16.00

Average Turnaround Time : 20.90

Average Response Time   : 16.00

CPU Utilization         : 100.00 %

Execution order (PID sequence): P1 P2 P3 P4 P1 P5 P6 P7 P8 P9 P10 P3 P4 P5 P7 P8 P10 P3 P7

----- Round Robin (Quantum = 3) : Result Table -----

PID	AT	BT	CT	WT	TAT	RT

P1	0	5	14	9	14	0

P2	1	3	6	2	5	2

P3	2	8	48	38	46	4

P4	3	6	37	28	34	6

P5	4	4	38	30	34	10

P6	5	2	19	12	14	12

P7	6	7	49	36	43	13

P8	7	5	43	31	36	15

P9	8	3	28	17	20	17

P10	9	6	46	31	37	19

Average Waiting Time    : 23.40

Average Turnaround Time : 28.30

Average Response Time   : 9.80

CPU Utilization         : 100.00 %

============== MODULE 2: DEADLOCK DETECTION / AVOIDANCE ==============

----- Banker's Algorithm : Deadlock Avoidance / Safety Check -----

Process	Allocation	Max		Need

P0	0 1 0 	7 5 3 	7 4 3

P1	2 0 0 	3 2 2 	1 2 2

P2	3 0 2 	9 0 2 	6 0 0

P3	2 1 1 	2 2 2 	0 1 1

P4	0 0 2 	4 3 3 	4 3 1

Available: 3 3 2

Result: SYSTEM IS IN A SAFE STATE

Safe Sequence: < P1 P3 P4 P0 P2 >

============== MODULE 3: MEMORY MANAGEMENT ==============

----- Paging Analysis -----

Page Size = 1024 KB | Total Frames Available = 12 (12288 KB)

Proc	Size(KB)	PagesNeeded	FrameAlloc(KB)	InternalFrag(KB)

P1	2200		3		3072		872

P2	3600		4		4096		496

P3	1200		2		2048		848

P4	4800		5		5120		320

P5	1500		2		2048		548

Total Frames Needed     : 16 (EXCEEDS available frames -> page-in/out & possible page faults)

Total Internal Fragmentation : 3084 KB

Memory Utilization           : 108.24 %

----- Segmentation Analysis (First-Fit on Variable Partitions) -----

Segment	Size(KB)	PartitionAlloc	PartSize(KB)	ExtFrag(KB)

S1	2200		Part-1		2500		300

S2	3600		Part-2		4000		400

S3	1200		Part-3		1300		100

S4	4800		Part-4		5000		200

S5	1500		Part-5		1600		100

Total External Fragmentation : 1100 KB

Unallocated Segments          : 0

Memory Utilization            : 92.36 %

============== MODULE 4: FILE ALLOCATION STRATEGIES ==============

----- File Allocation Strategy Comparison -----

Total Disk Blocks = 40 | Pre-occupied (fragmented) blocks = 10

Method		FilesAllocated	BlocksUsed	OverheadBlocks	AvgAccessCost(blocks)

Contiguous	4/6		18		0		1 (direct, seq. fast)

Linked		6/6		28		0		2.83 (must traverse chain)

Indexed		0/6		34		6		2 (index + 1 data block)

Contiguous: SOME FILES FAILED to allocate due to external fragmentation (10 holes pre-occupied on disk).

Linked   : No external fragmentation; scattered blocks are fine; poor random access.

Indexed  : No external fragmentation; 6 extra index blocks used; good random access.

============== MODULE 5: DISK SCHEDULING ==============

Requests (12): 82 95 30 63 12 70 25 48 90 5 77 34

Initial Head Position = 50 | Cylinders = 0-99

FCFS  Order: 50 -> 82 -> 95 -> 30 -> 63 -> 12 -> 70 -> 25 -> 48 -> 90 -> 5 -> 77 -> 34

FCFS  Total Head Movement = 562 | Avg Seek = 46.83

SSTF  Order: 50 -> 48 -> 34 -> 30 -> 25 -> 12 -> 5 -> 63 -> 70 -> 77 -> 82 -> 90 -> 95

SSTF  Total Head Movement = 135 | Avg Seek = 11.25

SCAN  Order: 50 -> 63 -> 70 -> 77 -> 82 -> 90 -> 95 -> 99(end) -> 48 -> 34 -> 30 -> 25 -> 12 -> 5

SCAN  Total Head Movement = 143 | Avg Seek = 11.92

C-SCAN Order: 50 -> 63 -> 70 -> 77 -> 82 -> 90 -> 95 -> 99(end) -> 0(jump) -> 5 -> 12 -> 25 -> 30 -> 34 -> 48

C-SCAN Total Head Movement = 196 | Avg Seek = 16.33

LOOK  Order: 50 -> 63 -> 70 -> 77 -> 82 -> 90 -> 95 -> 48 -> 34 -> 30 -> 25 -> 12 -> 5

LOOK  Total Head Movement = 135 | Avg Seek = 11.25

C-LOOK Order: 50 -> 63 -> 70 -> 77 -> 82 -> 90 -> 95 -> 5 -> 12 -> 25 -> 30 -> 34 -> 48

C-LOOK Total Head Movement = 178 | Avg Seek = 14.83

=====================================================================

END OF ANALYSIS

=====================================================================

# 5. Performance Calculation Tables

## 5.1 CPU Scheduling

| Algorithm | Avg Waiting Time | Avg Turnaround Time | Avg Response Time | CPU Utilization |
| --- | --- | --- | --- | --- |
| FCFS | 17.80 | 22.70 | 17.80 | 100% |
| SJF (NP) | 13.20 | 18.10 | 13.20 | 100% |
| Priority (NP) | 16.00 | 20.90 | 16.00 | 100% |
| Round Robin (q=3) | 23.40 | 28.30 | 9.80 | 100% |

## 5.2 Deadlock Analysis

| Available | Result | Safe Sequence |
| --- | --- | --- |
| 3 3 2 (A B C) | SAFE | P1 -> P3 -> P4 -> P0 -> P2 |

## 5.3 Memory Management

| Technique | Total Fragmentation | Memory Utilization | Notes |
| --- | --- | --- | --- |
| Paging | 3084 KB (internal) | 108.24% (demand/supply) | Needs 16 frames; only 12 available -> page faults likely |
| Segmentation | 1100 KB (external) | 92.36% | All 5 segments placed via first-fit |

## 5.4 File Allocation

| Method | Files Allocated | Blocks Used | Overhead Blocks | Avg Access Cost |
| --- | --- | --- | --- | --- |
| Contiguous | 4 / 6 | 18 | 0 | 1 block (fast, but fails under fragmentation) |
| Linked | 6 / 6 | 28 | 0 | 2.83 blocks (chain traversal) |
| Indexed | 0 / 6* | 34 | 6 | 2 blocks (index + data) |

*In this run, indexed allocation needs 34 blocks (28 data + 6 index) against 30 free blocks, so it could not fully complete on this constrained disk — illustrating the overhead cost of indexed allocation on small volumes.

## 5.5 Disk Scheduling

| Algorithm | Total Head Movement | Average Seek Time |
| --- | --- | --- |
| FCFS | 562 | 46.83 |
| SSTF | 135 | 11.25 |
| SCAN | 143 | 11.92 |
| C-SCAN | 196 | 16.33 |
| LOOK | 135 | 11.25 |
| C-LOOK | 178 | 14.83 |

# 6. Comparative Analysis

## 6.1 CPU Scheduling

- FCFS is simplest but produces high waiting time for long jobs arriving early (convoy effect visible for P3, BT=8).

- SJF (NP) gives the lowest average waiting/turnaround time (13.20 / 18.10) among non-preemptive methods but can starve long jobs like P3 (WT=39) and requires burst-time prediction, which is hard in a live data center.

- Priority Scheduling reflects business SLAs (e.g., banking > analytics) but a poor priority assignment can starve low-priority jobs (P6 waits 36 units).

- Round Robin gives the best (lowest) average response time (9.80) — critical for interactive/latency-sensitive services — at the cost of higher average waiting/turnaround time due to context switching between jobs.

## 6.2 Deadlock Analysis

- With Available = (3,3,2) the Banker's Algorithm found a valid safe sequence (P1→P3→P4→P0→P2), confirming the resource manager can grant all outstanding maximum claims without risking deadlock.

- This same check should run before granting any new resource request in the live data center (deadlock avoidance), not only as a one-time audit.

## 6.3 Memory Management

- Paging eliminates external fragmentation entirely but wastes 3084 KB to internal fragmentation and, in this workload, needs 16 frames against only 12 available — triggering page faults / swapping.

- Segmentation matches variable-sized partitions to logical process/segment sizes (better semantic fit) and keeps external fragmentation lower in relative terms (1100 KB, 92.36% utilization), but is vulnerable to fragmentation growing over time as partitions are freed and reallocated.

- For a data center running processes of very different, unpredictable sizes, Paging is generally preferred (predictable allocation, no external fragmentation) provided enough physical frames are provisioned; Segmentation suits workloads with well-defined logical modules (e.g., code/data/stack separation).

## 6.4 File Allocation

- Contiguous allocation gave the fastest access (1 seek) but only completed 4 of 6 files because of pre-existing external fragmentation — unsuitable for a busy data center disk without periodic compaction.

- Linked allocation allocated all 6 files (no external fragmentation) but has the worst random-access performance (avg 2.83 block hops) — poor fit for databases needing random reads.

- Indexed allocation gives good random access (2 blocks) with only modest overhead (1 block/file) but needs the most total space; on a nearly-full disk it can fail outright as shown here.

- For general-purpose data-center storage, Indexed allocation is the recommended default (predictable random access, no external fragmentation), provided the disk is not run near-full; Linked suits archival/sequential logs.

## 6.5 Disk Scheduling

- FCFS performed far worse than every other algorithm (562 total movement) because service order is arbitrary.

- SSTF and LOOK tied for the lowest head movement (135) for this request set, but SSTF can starve far-away requests under continuous load.

- SCAN/LOOK and C-SCAN/C-LOOK trade a little extra movement for more UNIFORM wait times across cylinders (avoids starvation) — C-SCAN/C-LOOK are preferred in high-load data-center storage where fairness matters more than raw minimum movement.

- Overall, LOOK is the best practical choice here: SSTF-level efficiency (135) without SSTF's starvation risk.

## 6.6 Charts

# 7. Final Recommendation with Justification

## Recommended Combination for this Data-Center Workload

| Layer | Recommended Technique | Justification |
| --- | --- | --- |
| CPU Scheduling | Round Robin (interactive tiers) + Priority (batch/SLA tiers) | Lowest response time (9.80) protects latency-sensitive banking/e-commerce requests; priority layer still respects SLA ordering for batch analytics. |
| Deadlock Handling | Banker's Algorithm (runtime avoidance) | Confirms safe state (P1→P3→P4→P0→P2) before granting resources; prevents circular-wait conditions among concurrent tenant processes. |
| Memory Management | Paging with adequate frame provisioning | Zero external fragmentation and predictable allocation; provision >=16 frames to remove the page-fault risk seen at 12 frames. |
| File Allocation | Indexed Allocation | Best random-access cost (2 blocks) with only 1 block/file overhead and no external fragmentation; keep disk utilization <85% to avoid the space failure observed near-full. |
| Disk Scheduling | LOOK (or C-LOOK under heavy sustained load) | Matches SSTF's minimum head movement (135) while avoiding starvation of distant requests; C-LOOK trades some movement for more uniform service time under continuous heavy I/O. |

Justification Summary: The recommendation is driven by CPU utilization (all algorithms achieved 100%, so response time became the deciding factor), memory utilization and fragmentation (Paging removes external fragmentation entirely), deadlock safety (Banker's Algorithm certifies a safe sequence under the current Available vector), storage efficiency (Indexed allocation balances access speed against modest overhead), and disk head movement (LOOK ties the best-performing algorithm, SSTF, without its starvation drawback). Together these choices minimize latency for customer-facing banking/e-commerce workloads while keeping storage and memory subsystems fragmentation-free and deadlock-safe.

# 8. Reflection on Design Decisions and Challenges

- Using ONE shared dataset across every technique in a component (e.g., the same 10 processes for FCFS/SJF/Priority/RR) was essential for a fair, apples-to-apples comparison, but required careful design of a workload that exercises every algorithm's strengths and weaknesses (e.g., a mix of short and long bursts to reveal convoy effects and starvation).

- Implementing Round Robin correctly (handling new arrivals that occur mid-quantum, and re-queuing a partially executed process) was the trickiest part of the CPU scheduler and required a dedicated event-ordered queue simulation rather than simple sorting.

- For file allocation, a purely theoretical comparison would not show WHY contiguous allocation is risky; deliberately pre-fragmenting the simulated disk (10 scattered occupied blocks) made the external-fragmentation failure of contiguous allocation concrete and measurable rather than just descriptive.

- An early version of the Banker's Algorithm module had a subtle array-copying bug (row-stride mismatch when copying a 5x3 matrix into a wider 20x5 buffer) that silently corrupted the Allocation/Max matrices; this was caught by cross-checking the output against the well-known Silberschatz textbook example, reinforcing the importance of validating simulation output against known results.

- A key design trade-off was choosing non-preemptive SJF/Priority (simpler, matches most course syllabi) instead of preemptive SRTF/Priority; a natural extension would add preemptive variants and multilevel feedback queues for a more realistic data-center scheduler.

- Overall, the exercise showed that no single technique is universally best — the right choice depends on workload shape (burst-time variance, resource contention, request locality) — which is exactly why the system is designed to be data-driven and re-runnable on different workloads rather than hard-coded to one 'best' algorithm.

# 9. GitHub Upload Note

The complete source code (datacenter.c) compiles with: gcc -o datacenter datacenter.c -lm

Recommended repository structure for submission:

/datacenter-optimizer

|- datacenter.c          (full source, Section 3 of this report)

|- output.txt            (sample run output, Section 4.2)

|- README.md             (problem statement + how to build/run)

|- report.docx           (this report)